| ID | Risk | Cause | Impact | Likelihood | Impact | Score | Mitigation | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 | ESP32 resets/brownouts | LED strip draws high current on same supply | Random failures, unusable demo | 3 | 5 | 15 | Separate LED power rail + common ground; add capacitors | Closed |
| R2 | LED overheating | High brightness in enclosed shell | Safety risk, component damage | 2 | 5 | 10 | Venting, heatsinking, brightness cap, thermal test | Mitigated |
| R3 | Motion false positives/negatives | PIR placement/angle, sensor noise | Wrong sleep detection, annoying behaviour | 3 | 3 | 9 | Calibrate threshold, debounce logic, test at distances | Open |
| R4 | WiFi setup friction | Users can’t connect | Can’t configure lamp | 3 | 3 | 9 | Captive portal via WiFiManager; clear instructions, manual buttons | Closed |
| R5 | Storage limits | ESP32 ~4MB flash | UI/assets/config crowd out space | 2 | 3 | 6 | Compress assets, limit logs, careful partitioning | Mitigated |
| R6 | Component delays/failures | Missing sensors/LED strip issues | Schedule slip | 3 | 2 | 6 | Order early, keep backup parts, mock sensors in code | Closed |
| R7 | Scope creep (apps) | iOS/Android integration expands | Project overruns | 4 | 3 | 12 | Define MVP: lamp + web UI first; app optional | Open |